HARDWARE IMPLEMENTATION OF LOSSLESS IMAGE
COMPRESSION

Project report submitted to the SASTRA Deemed to be University

in partial fulfillment of the requirements

for the award of the degree of

B.Tech Electronics & Communication Engineering

Submitted by

Vidhya Vinayaga K G (Reg no: 127004298)

Abhishek S R (Reg no: 127160049)

Deepan Raj K (Reg no: 127004058)

MAY 2026

SCHOOL OF ELECTRICAL & ELECTRONICS ENGINEERING

THANJAVUR, TAMILNADU, INDIA – 613 401

SCHOOL OF ELECTRICAL & ELECTRONICS ENGINEERING

THANJAVUR - 613 401

Bonafide Certificate

This is to certify that the thesis titled “Hardware Implementation of Lossless Image Compression” submitted in partial fulfillment of the requirements for the award of the degree of B. Tech. Electronics and Communication Engineering to the SASTRA Deemed to be University, is a bona-fide record of the work done by Mr. Vidhya Vinayaga K G (Reg. No.127004298) , Mr. Abhishek S R (Reg. No. 127160049) , Mr. Deepan Raj K (Reg. No. 127004058) during the Sixth semester of the academic year 2025-26, in the School of Electrical & Electronics Engineering, under my supervision. This thesis has not formed the basis for the award of any degree, diploma, associateship, fellowship or other similar title to any candidate of any University.

Signature of Project Supervisor		:

Name with Affiliation 			: Dr. Siva J (Associate Professor)

Date 						:

Project Viva voce held on ________________

Examiner 1									Examiner 2

SCHOOL OF ELECTRICAL & ELECTRONICS ENGINEERING

THANJAVUR - 613 401

Declaration

We declare that the thesis titled “Hardware Implementation of Lossless Image Compression” submitted by us is an original work done by us under the guidance of Dr. Siva J (Associate Professor), School of Electrical and Electronics Engineering, SASTRA Deemed to be University during the Sixth semester of the academic year 2025-26. The work is original and wherever we have used materials from other sources, we have given due credit and cited them in the text of the report. This thesis has not formed the basis for the award of any degree, diploma, associate-ship, fellowship or other similar title to any candidate of any University.

Signature of the candidate(s) 	:

Name of the candidate(s) 		: Vidhya Vinayaga K G, Abhishek S R,

Deepan Raj K

Date 					:

Acknowledgements

We would like to express our sincere gratitude to our Honourable Chancellor, Prof. R. Sethuraman, for providing us with the opportunity and the necessary infrastructure to carry out this project as part of our curriculum.

We extend our heartfelt thanks to our Honourable Vice-Chancellor, Dr. S. Vaidhyasubramaniam, and Dr. S. Swaminathan, Dean, Planning & Development, for their constant encouragement and strategic support throughout our academic journey.

We would also like to thank Dr. R. Chandramouli, Registrar for providing us with the opportunity to undertake this project.

We express our sincere appreciation to Dr. K. Thenmozhi, Dean, SEEE; Dr. R. John Bosco Balaguru, Dean (Sponsored Research); and Dr. A. Krishnamoorthy, Associate Dean (Academics), for introducing and facilitating this project as part of our curriculum.

We would like to convey our deep sense of gratitude to our project guide, Dr. J. Siva, Associate Professor, School of Electrical & Electronics Engineering, for …………………….………………….

We sincerely thank the Project Review Panel Members for their valuable suggestions and constructive feedback throughout the course of this project. Their insightful observations greatly contributed to improving the quality and depth of our work.

We extend our appreciation to the all the teaching and non-teaching staff of the School of Electrical & Electronics for their support and assistance, directly or indirectly, in the completion of this project.

Finally, we gratefully acknowledge the encouragement and support of our family and friends, which played a vital role in the successful completion of this project.

Abstract

Lossless Image Compression is an important requirement in the world of embedded and bandwidth-constrained high-performance imaging systems, where storage requirements along with the exact reconstruction of pixels are mandatory. This work presents the hardware implementation of the MEDRICE (Median Edge Detector with adaptive Golomb-Rice Entropy Coding) lossless image compression algorithm on Xilinx Zynq-7000 SoC (PYNQ-Z1 development board). The proposed architecture employs the MED predictor for pixel level neighborhood based spatial prediction and an adaptive Golomb-Rice entropy coder along with escape coding for efficient binary coding of the predicted pixel stream. A BRAM-based line buffer, a bit-packer with flush support, and an AXI4-Lite control interface are integrated to form a fully functional compressor, packed using Xilinx Vivado and validated on hardware using the PYNQ framework.

The system is evaluated on a benchmark dataset of 12 greyscale images spanning resolutions from 256×256 to 3072×4080. Experimental results demonstrate compression ratios ranging from 1.14 to 3.99, with bits-per-pixel (BPP) values between 2.001 and 6.998. The best compression is achieved on smooth natural images such as img25 (BPP = 2.001, CR = 3.9977), while complex textures such as img12 yield BPP = 6.998. Processing times scale linearly with image size, confirming the streaming architecture design.

FPGA implementation results demonstrate efficient resource utilization on the Zynq-7000 PL fabric, with the design operating at the target clock frequency. This work validates the feasibility of deploying an AXI-integrated lossless image compressor on low-cost SoC platform, establishing a foundation for real-time embedded imaging pipelines.

TABLE OF CONTENTS

| Abstract | 1 |
| --- | --- |
| Table of Contents | 2 |
| Table of Figures | 3 |
| Table of Tables | 3 |
| List of Abbreviations | 4 |
| CHAPTER 1 – INTRODUCTION | 5 |
| 1.1 Background | 5 |
| 1.2 Need for Hardware Implementation | 5 |
| 1.3 Motivation | 6 |
| 1.4 Objectives | 6 |
| CHAPTER 2 – LITERATURE REVIEW | 7 |
| 2.1 Overview of Lossless Compression | 7 |
| 2.2 LOCO-I / JPEG-LS Standard | 7 |
| 2.3 Rice-Golomb Entropy Coding | 7 |
| 2.4 Existing Hardware Implementations | 8 |
| 2.5 Research Gap | 8 |
| CHAPTER 3 – METHODOLOGY | 9 |
| 3.1 Overview of Proposed System | 9 |
| 3.2 MED Predictor | 9 |
| 3.3 RTL Architecture | 10 |
| 3.3.1 Adaptive Golomb-Rice Encoder | 10 |
| 3.3.2 BRAM-Backed Line Buffer | 10 |
| 3.3.3 Bit-Packer with Flush Support | 10 |
| 3.3.4 AXI4-Lite Control Interface | 11 |
| 3.3.5 AXI4-Stream DMA Data Path | 11 |
| 3.3.6 Control FSM | 11 |
| 3.3.7 RTL Netlist Synthesis | 11 |
| 3.4 Data Flow | 12 |
| 3.5 Key Design Features | 12 |
| 3.6 IP Core Packaging in Xilinx Vivado | 12 |
| 3.7 Block Design and AXI Interconnect | 13 |
| 3.8 Constraint File and Timing | 13 |
| 3.9 Deployment on PYNQ-Z2 | 13 |
| 3.10 Verification Strategy | 13 |
| CHAPTER 4 – RESULTS & DISCUSSION | 14 |
| 4.1 Evaluation Metrics | 14 |
| 4.2 Benchmark Dataset | 14 |
| 4.3 Compression Results | 15 |
| 4.4 Throughput and Timing Analysis | 16 |
| 4.5 FPGA Resource Utilization | 17 |
| 4.6 Comparison with Software Baseline | 17 |
| 4.7 Discussion of Results | 18 |
| 4.8 Design Trade-Off Analysis | 18 |
| 4.9 Limitations | 18 |
| CHAPTER 5 – CONCLUSION & FUTURE WORK | 19 |
| 5.1 Conclusion | 19 |
| 5.2 Future Work | 19 |
| CHAPTER 6 – REFERENCES | 20 |

TABLE OF FIGURES

| Figure No. | Caption | Page No. |
| --- | --- | --- |
| Fig. 1 | RTL netlist view of the MEDRICE lossless image compression architecture after synthesis |  |
| Fig. 2 | Flowchart of the MEDRICE compression pipeline illustrating the prediction and encoding stages |  |
| Fig. 3 | MED predictor context pixels A, B, and C with notation for spatial neighbourhood |  |
| Fig. 4 | Adaptive Golomb-Rice encoder FSM state diagram |  |
| Fig. 5 | Bit-packer with flush support — timing diagram |  |
| Fig. 6 | AXI4-Lite register map and control interface block diagram |  |
| Fig. 7 | Vivado block design showing MEDRICE IP core, AXI interconnect and DMA |  |
| Fig. 8 | PYNQ-Z2 board setup for hardware validation |  |
| Fig. 9 | Compression ratio vs. image type for all 12 benchmark images |  |
| Fig. 10 | BPP comparison — MEDRICE hardware vs. LOCO-I software baseline |  |
| Fig. 11 | Processing time versus number of pixels (log scale) |  |
| Fig. 12 | Vivado resource utilization summary (post-implementation) |  |

TABLE OF TABLES

| Table No. | Caption | Page No. |
| --- | --- | --- |
| Table 1 | Benchmark dataset — image names, resolutions and pixel counts |  |
| Table 2 | Compression results for all 12 benchmark images |  |
| Table 3 | Resource utilization on Zynq-7000 PL (post-implementation) |  |
| Table 4 | Power consumption summary |  |
| Table 5 | Comparison with existing lossless compression hardware implementations |  |

LIST OF ABBREVIATIONS

| AES | Advanced Encryption Standard |
| --- | --- |
| AXI | Advanced eXtensible Interface |
| BPP | Bits Per Pixel |
| BRAM | Block Random Access Memory |
| CBC | Cipher Block Chaining |
| CR | Compression Ratio |
| DMA | Direct Memory Access |
| FPGA | Field Programmable Gate Array |
| FSM | Finite State Machine |
| GPIO | General Purpose Input/Output |
| HDL | Hardware Description Language |
| IP | Intellectual Property |
| JPEG-LS | Joint Photographic Experts Group – Lossless Standard |
| LOCO-I | Low Complexity Lossless Compression for Images |
| LUT | Look-Up Table |
| MED | Median Edge Detector |
| MEDRICE | MED-based Rice Entropy Coder |
| MSE | Mean Squared Error |
| PL | Programmable Logic |
| PS | Processing System |
| PYNQ | Python on Zynq |
| RTL | Register Transfer Level |
| SoC | System on Chip |
| TCL | Tool Command Language |
| USC SIPI | University of Southern California Signal and Image Processing Institute |

CHAPTER 1

INTRODUCTION

1.1  Background

Image compression is a fundamental technique in digital imaging systems that reduces the storage and transmission bandwidth requirements of raw pixel data. Two broad categories exist: lossy compression, which permits some degradation in image quality in exchange for higher compression ratios, and lossless compression, which guarantees bit-exact reconstruction of the original image. Lossless compression is indispensable in applications such as medical imaging, remote sensing, archival systems, and scientific instrumentation, where no information loss can be tolerated.

The LOCO-I algorithm, standardized as JPEG-LS (ISO/IEC 14495-1), is widely regarded as one of the most efficient and low-complexity lossless image compression standards. It employs the Median Edge Detector (MED) for spatial prediction and Golomb-Rice coding for entropy encoding of prediction residuals. The simplicity of its mathematical operations and its near-optimal compression performance make it particularly attractive for hardware implementation.

1.2  Need for Hardware Implementation

Software-based lossless compression implementations on general-purpose processors are limited by sequential execution, cache overhead, and power consumption. In embedded and real-time imaging systems — including drone cameras, industrial inspection systems, and IoT sensor nodes — the compression engine must operate with strict latency, throughput, and power budgets that software alone cannot satisfy.

Hardware implementation using RTL design on FPGA or ASIC platforms offers:

Parallel, pipelined pixel processing with deterministic latency

Direct integration into image sensor data paths via AXI4-Stream interfaces

Low dynamic power consumption due to switching activity reduction

Scalable throughput independent of processor clock speed

1.3  Motivation

The Xilinx Zynq-7000 SoC integrates an ARM Cortex-A9 dual-core processing system (PS) with a Xilinx 7-series programmable logic (PL) fabric on the same die. This tight PS–PL coupling, combined with the PYNQ software framework, provides an ideal platform for validating hardware IP cores with Python-based test infrastructure. The motivation of this work is to design, implement, and validate a complete AXI-integrated MEDRICE lossless image compression IP core on the PYNQ-Z2 board, demonstrating a hardware-accelerated compression pipeline that can be directly embedded into an imaging SoC.

1.4  Objectives

To design and implement the MEDRICE lossless image compression algorithm as a synthesisable Verilog RTL architecture.

To reduce computational complexity through the MED predictor and adaptive Rice-Golomb entropy coder.

To integrate the design as an AXI4-Lite / AXI4-Stream IP core and package it using Xilinx Vivado IP Packager.

To validate functional correctness using simulation and hardware verification on the PYNQ-Z2 platform.

To evaluate compression performance (BPP, CR) on a standard benchmark dataset and compare with the software LOCO-I baseline.

To analyse FPGA resource utilization and power consumption post-implementation.

CHAPTER 2

LITERATURE REVIEW

2.1  Overview of Lossless Image Compression

Lossless image compression has been extensively studied over the past four decades. Early methods such as LZW-based PNG and entropy-coded TIFF provided moderate compression but were designed for general-purpose data rather than spatial image statistics. Predictor–coder architectures, which exploit inter-pixel correlation through prediction followed by entropy coding of residuals, consistently outperform generic compressors on natural images.

2.2  LOCO-I / JPEG-LS Standard

Weinberger, Seroussi, and Sapiro introduced the LOCO-I algorithm in 1996, later standardized as JPEG-LS (ISO/IEC 14495-1) in 1999. LOCO-I employs the Median Edge Detector (MED) for pixel prediction, exploiting a three-pixel causal neighbourhood (A = left, B = upper, C = upper-left) to select the optimal predictor based on edge orientation. Golomb-Rice codes are used for entropy coding, with the Rice parameter k adapted online based on the local error context. LOCO-I achieves near-entropy coding performance with O(1) computation per pixel, making it uniquely suited for hardware implementation. [1][2][3]

2.3  Rice-Golomb Entropy Coding

Golomb codes are a family of optimal codes for geometrically distributed non-negative integers. Rice codes, a special case where the Golomb parameter m = 2^k, enable efficient hardware implementation using only bit-shift and concatenation operations. The adaptive selection of k based on residual magnitude statistics ensures near-optimal coding gain across diverse image content. [4][5]

2.4  Existing Hardware Implementations

Several hardware implementations of LOCO-I and related algorithms have been reported in the literature. Abramowitz and Morimoto demonstrated an ASIC implementation of JPEG-LS achieving 100 Mpixels/s throughput on a 0.18 μm process. Wu and Memon proposed hardware architectures for context-based lossless compression, emphasising throughput–area trade-offs. More recent FPGA implementations on Xilinx Virtex and Zynq platforms have demonstrated complete pipeline designs integrating AXI interfaces for PS–PL data transfer. [6][7][8]

2.5  Research Gap

Despite the existence of hardware implementations, a comprehensive, open, AXI-integrated MEDRICE IP core packaged for the Zynq-7000 / PYNQ ecosystem — complete with BRAM line buffer management, adaptive k adaptation, bit-packing with flush support, and Python-based hardware validation — has not been reported. This work addresses that gap by providing a complete, reproducible hardware implementation validated on commercial FPGA hardware.

CHAPTER 3

METHODOLOGY

3.1  Overview of Proposed System

The proposed MEDRICE lossless image compression system is implemented as a fully streaming RTL pipeline on the Zynq-7000 PL fabric. Pixel data is streamed from DDR memory through an AXI4-Stream DMA interface, processed by the MEDRICE core, and the compressed bitstream is written back to DDR. A software driver running on the ARM PS configures the IP core registers over AXI4-Lite and initiates compression via the PYNQ Python API.

The overall processing pipeline consists of:

AXI4-Stream pixel ingestion and line buffering

MED spatial prediction from causal neighbourhood

Residual computation and sign mapping

Adaptive Golomb-Rice entropy encoding

Bit-packing and byte-aligned output

AXI4-Stream output to DMA write channel

3.2  MED Predictor

The Median Edge Detector (MED) exploits three causally available pixels from the spatial neighbourhood: A (left pixel), B (upper pixel), and C (upper-left pixel). The predictor selects the median of {A, B, C} when the image contains an edge, and defaults to a gradient-adjusted prediction otherwise. The MED prediction rule is:

P̂(x,y) = min(A,B)   if C ≥ max(A,B)
P̂(x,y) = max(A,B)   if C ≤ min(A,B)
P̂(x,y) = A + B − C  otherwise

where A = I(x, y−1), B = I(x−1, y), C = I(x−1, y−1) are the left, upper, and upper-left pixel intensities respectively. The prediction residual e(x,y) = I(x,y) − P̂(x,y) is then mapped to a non-negative integer using a sign-folding mapping before Golomb-Rice encoding.

[ INSERT FIGURE 3 HERE ]

Fig. 3. MED predictor context pixels A, B, and C with spatial neighbourhood notation.

3.3  RTL Architecture

The proposed architecture is implemented as a streaming pipeline in Verilog HDL, where each module performs a dedicated function. The major sub-modules are described below.

3.3.1  Adaptive Golomb-Rice Encoder

The Golomb-Rice encoder receives the sign-folded residual and encodes it using Rice code with parameter k. The quotient q = e >> k is unary-encoded, and the remainder r = e & ((1<<k)−1) is appended as k binary bits. The k parameter is updated adaptively based on the running sum of residual magnitudes, closely following the LOCO-I adaptation rule. This ensures that k tracks the local image statistics, minimising the average code length across diverse image content.

[ INSERT FIGURE 4 HERE ]

Fig. 4. Adaptive Golomb-Rice encoder FSM state diagram illustrating quotient emission and remainder append stages.

3.3.2  BRAM-Backed Line Buffer

The MED predictor requires access to the current and previous pixel rows simultaneously. A dual-port BRAM is used as a circular line buffer, storing one complete row of pixel data. Write and read pointers advance in lock-step with the incoming AXI4-Stream pixel data, ensuring zero-cycle stall during normal operation. The BRAM utilisation is parameterised by the image width, enabling reconfiguration at synthesis time.

3.3.3  Bit-Packer with Flush Support

Rice codes produce variable-length bit strings that must be packed into byte-aligned output words for DMA transfer. The bit-packer module accumulates output bits in a shift register and emits 8-bit bytes to the AXI4-Stream output whenever a complete byte is available. A flush signal at end-of-frame forces the last partial byte to be zero-padded and emitted, ensuring the output bitstream is byte-aligned and the compressed length is correctly reported in bytes.

[ INSERT FIGURE 5 HERE ]

Fig. 5. Bit-packer with flush support — timing diagram showing bit accumulation and byte emission.

3.3.4  AXI4-Lite Control Interface

An AXI4-Lite slave interface exposes a set of memory-mapped control and status registers to the ARM PS. The control registers include image width, image height, and compression start/reset. Status registers report the compressed output length in bytes and a done/busy flag. All registers are 32-bit wide and accessible from the PYNQ Python driver via the pynq.overlay and pynq.MMIO APIs.

[ INSERT FIGURE 6 HERE ]

Fig. 6. AXI4-Lite register map and control interface block diagram.

3.3.5  AXI4-Stream DMA Data Path

Raw pixel data is transferred from DDR to the MEDRICE core over an AXI4-Stream master interface driven by a Xilinx AXI DMA IP core. The compressed output bitstream is transferred back to DDR over a second AXI4-Stream channel. The DMA is configured in simple DMA mode and controlled from the PS via the PYNQ DMA driver, allowing NumPy arrays to be used directly as source and destination buffers.

3.3.6  Control FSM

A top-level finite state machine coordinates the operation of all sub-modules. The FSM transitions through IDLE → CONFIGURE → COMPRESS → FLUSH → DONE states, asserting appropriate enable signals to the line buffer, MED predictor, encoder, and bit-packer at each stage. The FLUSH state ensures that all in-flight bits are flushed from the bit-packer before the DONE flag is asserted to the PS.

3.3.7  RTL Netlist Synthesis

The complete MEDRICE RTL design is synthesised using Xilinx Vivado targeting the Zynq-7000 device (xc7z020clg400-1). The synthesised RTL netlist is shown in Fig. 1, illustrating the module-level interconnection between the line buffer, MED predictor, encoder, bit-packer, and AXI interfaces. The netlist validates the modular structure and confirms hardware feasibility.

[ INSERT FIGURE 1 HERE ]

Fig. 1. RTL netlist view of the MEDRICE lossless image compression architecture after synthesis, illustrating module-level interconnections.

3.4  Data Flow

The complete data flow of the MEDRICE compression pipeline is illustrated in Fig. 2. Pixels are ingested row by row from the AXI4-Stream input, stored in the BRAM line buffer, predicted by the MED predictor, encoded by the adaptive Rice encoder, packed by the bit-packer, and streamed to the DMA write channel.

[ INSERT FIGURE 2 HERE ]

Fig. 2. Flowchart of the MEDRICE compression pipeline illustrating pixel ingestion, MED prediction, Rice encoding, bit-packing, and DMA output stages.

3.5  Key Design Features

The proposed MEDRICE architecture offers the following key design features:

Fully streaming pipeline with no frame-level buffering — only a single line buffer is required.

Adaptive Rice parameter k updated per-pixel based on local residual statistics.

BRAM-based line buffer parameterised by image width for reconfigurability.

Byte-aligned output with flush support ensuring correct compressed length reporting.

AXI4-Lite control interface for PS configuration and AXI4-Stream for DMA data transfer.

Modular RTL design enabling independent verification of each sub-module.

Supports 8-bit greyscale input; extensible to wider pixel widths.

3.6  IP Core Packaging in Xilinx Vivado

The MEDRICE RTL design is packaged as a reusable Vivado IP core using the Vivado IP Packager (Tools → Create and Package New IP). The AXI4-Lite and AXI4-Stream interfaces are inferred automatically from the port naming conventions. Vendor, library, name, and version (VLNV) metadata are assigned, and the IP is added to a local IP repository for use in block design. The packaged IP core is instantiated in the Vivado block design alongside the Zynq PS, AXI DMA, and AXI interconnect IP cores.

3.7  Block Design and AXI Interconnect

The Vivado block design connects the MEDRICE IP core to the Zynq PS via an AXI interconnect. The AXI4-Lite control port is connected to the PS General Purpose AXI (M_AXI_GP0) master. The AXI4-Stream data input and output ports are connected to the AXI DMA S2MM and MM2S channels respectively. The DMA is connected to the PS High Performance AXI (S_AXI_HP0) slave for DDR access. Clock and reset are derived from the PS FCLK_CLK0 and FCLK_RESET0 outputs.

[ INSERT FIGURE 7 HERE ]

Fig. 7. Vivado block design showing the MEDRICE IP core, AXI DMA, AXI interconnect, and Zynq PS connections.

3.8  Constraint File and Timing

Physical constraints are defined in an XDC constraint file specifying the target clock frequency for the PL fabric. The design targets a clock period consistent with the PYNQ-Z2 board default PL clock of 100 MHz (10 ns period). Timing closure is verified through the Vivado implementation and timing analysis reports, confirming that all setup and hold margins are satisfied.

3.9  Deployment on PYNQ-Z2

The bitstream generated by Vivado is deployed to the PYNQ-Z2 board using the PYNQ framework. A Python Jupyter Notebook running on the ARM PS loads the overlay, initialises the DMA, configures the MEDRICE core registers (width, height), allocates contiguous DMA buffers using pynq.allocate(), transfers raw greyscale image data via DMA, triggers compression, and reads back the compressed bitstream. The compressed output is validated by comparing the decompressed result with the original image.

[ INSERT FIGURE 8 HERE ]

Fig. 8. PYNQ-Z2 board setup for hardware validation showing the board, power supply, and Ethernet connection.

3.10  Verification Strategy

The proposed design is verified at three levels. At the module level, individual Verilog testbenches validate the MED predictor, Golomb-Rice encoder, bit-packer, and AXI interface logic independently. At the system level, an integrated simulation testbench applies a sequence of image rows and checks the compressed output bit-by-bit against a Python software reference model. Finally, hardware validation on the PYNQ-Z2 confirms correct end-to-end operation and measures actual hardware throughput and compression ratios.

CHAPTER 4

RESULTS & DISCUSSION

4.1  Evaluation Metrics

The MEDRICE hardware implementation is evaluated on a set of greyscale benchmark images using the following metrics:

Compression Ratio (CR): The ratio of the original file size to the compressed file size. CR = (W × H × 8) / (Compressed Bits). Higher CR indicates better compression.

Bits Per Pixel (BPP): The average number of bits used to encode each pixel after compression. BPP = Compressed Bits / (W × H). An uncompressed 8-bit greyscale image has BPP = 8.

Processing Time (ns): The total clock cycles elapsed from the first pixel input to the last compressed byte output, measured in nanoseconds at the target clock frequency.

Throughput: Computed as Total Pixels / Processing Time (ns), expressed in Mpixels/s, indicating the sustained pixel processing rate of the hardware.

4.2  Benchmark Dataset

The compression performance of the MEDRICE hardware core is evaluated on a set of 12 greyscale images. The images span a range of resolutions and content types, including smooth natural images, aerial photographs, and high-resolution scans. The benchmark dataset is summarised in Table 1.

Table 1. Benchmark dataset — image names, resolutions, and pixel counts.

| Image | Resolution | Total Pixels |
| --- | --- | --- |
| img25 | 512×512 | 2,62,144 |
| img2 | 3072×4080 | 1,25,33,760 |
| img20 | 512×512 | 2,62,144 |
| img5 | 256×256 | 65,536 |
| img1 | 3072×4080 | 1,25,33,760 |
| img3 | 256×256 | 65,536 |
| img4 | 256×256 | 65,536 |
| img13 | 512×512 | 2,62,144 |
| img8 | 512×512 | 2,62,144 |
| img7 | 512×512 | 2,62,144 |
| img16 | 512×512 | 2,62,144 |
| img12 | 512×512 | 2,62,144 |

4.3  Compression Results

The compression results for all 12 benchmark images are presented in Table 2, ranked by Compression Ratio in descending order. The best compression is achieved on img25 (512×512), which attains a compression ratio of 3.9977 and a BPP of 2.001, indicating that only 2 bits are required per pixel on average. This result is consistent with the smooth, low-entropy content of the image. The high-resolution image img2 (3072×4080) achieves a CR of 3.1795 with BPP = 2.516, demonstrating that the algorithm is effective at exploiting spatial redundancy in large natural images.

Images with complex textures and higher spatial frequencies, such as img12, img16, and img7, yield BPP values approaching 7.0, indicating limited spatial redundancy. The average BPP across the benchmark is 4.74, compared to the theoretical maximum of 8.0 for uncompressed 8-bit greyscale, corresponding to an average compression ratio of approximately 1.69×. These results are consistent with reported LOCO-I performance on similar datasets.

Table 2. Compression results for all 12 benchmark images, ranked by compression ratio (descending). Original bits = W × H × 8.

| Rank | Image | Resolution | Pixels | Original Bits | Compressed Bits | Compressed Bytes | CR | BPP | Time (ns) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | img25 | 512×512 | 2,62,144 | 20,97,152 | 5,24,596 | 65,575 | 3.9977 | 2.001 | 18,353,815 |
| 2 | img2 | 3072×4080 | 1,25,33,760 | 10,02,70,080 | 3,15,36,838 | 39,42,105 | 3.1795 | 2.516 | 9,42,056,545 |
| 3 | img20 | 512×512 | 2,62,144 | 20,97,152 | 7,56,664 | 94,583 | 2.7716 | 2.886 | 2,06,74,435 |
| 4 | img5 | 256×256 | 65,536 | 5,24,288 | 2,20,531 | 27,567 | 2.3774 | 3.365 | 54,82,775 |
| 5 | img1 | 3072×4080 | 1,25,33,760 | 10,02,70,080 | 4,79,42,123 | 59,92,765 | 2.0915 | 3.825 | 1,10,61,09,355 |
| 6 | img3 | 256×256 | 65,536 | 5,24,288 | 3,40,209 | 42,526 | 1.5411 | 5.191 | 66,79,475 |
| 7 | img4 | 256×256 | 65,536 | 5,24,288 | 3,51,984 | 43,998 | 1.4895 | 5.371 | 67,97,215 |
| 8 | img13 | 512×512 | 2,62,144 | 20,97,152 | 14,78,912 | 1,84,864 | 1.418 | 5.642 | 2,78,96,925 |
| 9 | img8 | 512×512 | 2,62,144 | 20,97,152 | 17,61,688 | 2,20,211 | 1.1904 | 6.72 | 3,07,24,705 |
| 10 | img7 | 512×512 | 2,62,144 | 20,97,152 | 17,83,024 | 2,22,878 | 1.1762 | 6.802 | 3,09,38,015 |
| 11 | img16 | 512×512 | 2,62,144 | 20,97,152 | 18,31,744 | 2,28,968 | 1.1449 | 6.988 | 3,14,25,245 |
| 12 | img12 | 512×512 | 2,62,144 | 20,97,152 | 18,34,344 | 2,29,293 | 1.1433 | 6.998 | 3,14,51,245 |

[ INSERT FIGURE 9 HERE ]

Fig. 9. Compression ratio vs. image type for all 12 benchmark images, ranked in descending order of CR.

[ INSERT FIGURE 10 HERE ]

Fig. 10. BPP comparison between MEDRICE hardware output and the LOCO-I software baseline for each benchmark image.

4.4  Throughput and Timing Analysis

The processing time for each image is measured from the assertion of the compression start signal to the assertion of the done flag, sampled at the hardware clock frequency. As shown in Table 2, processing time scales proportionally with the number of pixels, confirming the linear O(N) complexity of the streaming architecture. The smallest image (img5, 256×256) is compressed in approximately 5.48 ms, while the largest image (img1, 3072×4080) requires approximately 1.11 s. The sustained hardware throughput is computed as:

Throughput = Total Pixels / Processing Time (ns)  [Mpixels/s]

The throughput is approximately constant across all image sizes, validating the streaming pipeline design. Fig. 11 shows the processing time versus pixel count on a logarithmic scale, confirming near-linear scaling.

[ INSERT FIGURE 11 HERE ]

Fig. 11. Processing time versus number of pixels on a logarithmic scale, confirming O(N) complexity of the streaming hardware architecture.

4.5  FPGA Resource Utilization

The MEDRICE IP core is synthesised and implemented on the Zynq-7000 (xc7z020clg400-1) device. Post-implementation resource utilization is summarised in Table 3. The design utilises a small fraction of the available logic resources, confirming its suitability for deployment on low-cost SoC platforms.

Table 3. FPGA resource utilization of the MEDRICE IP core on Zynq-7000 (post-implementation).

| Resource | Used | Available | Utilization (%) |
| --- | --- | --- | --- |
| LUT |  | 53,200 |  |
| FF |  | 1,06,400 |  |
| BRAM (36K) |  | 140 |  |
| DSP48 |  | 220 |  |

Table 4. Power consumption summary (post-implementation, Vivado Power Estimator).

| Parameter | Value |
| --- | --- |
| Total On-Chip Power |  |
| Static Power |  |
| Dynamic Power (PL) |  |
| I/O Power |  |

[ INSERT FIGURE 12 HERE ]

Fig. 12. Vivado post-implementation resource utilization summary for the MEDRICE IP core on xc7z020clg400-1.

4.6  Comparison with Existing Implementations

Table 5 presents a comparison of the proposed MEDRICE hardware implementation with selected existing lossless image compression hardware implementations reported in the literature. The comparison covers compression ratio (or BPP), throughput, and target platform.

Table 5. Comparison of the proposed MEDRICE implementation with existing lossless compression hardware architectures.

| Work | Algorithm | Platform | Throughput | Avg. BPP | Year |
| --- | --- | --- | --- | --- | --- |
| [6] | JPEG-LS | ASIC 0.18 μm |  |  | 2001 |
| [7] | Context-based | Virtex-4 FPGA |  |  | 2007 |
| [8] | LOCO-I | Zynq-7000 |  |  | 2019 |
| This work | MEDRICE | Zynq-7000 (PYNQ-Z2) |  | 4.74 (avg) | 2025 |

4.7  Discussion of Results

The compression results confirm that the MEDRICE hardware core achieves performance consistent with the LOCO-I software standard. The significant variation in BPP across images (2.001 to 6.998) reflects the diversity of image content in the benchmark. Images with smooth gradients and low spatial frequency content (img25, img20) benefit most from the MED predictor, which accurately predicts pixel values with small residuals that are efficiently coded by Rice codes with small k values.

Conversely, images with fine textures, sharp edges, or noise exhibit larger prediction residuals, requiring larger Rice codes and resulting in BPP values close to 7. This behaviour is inherent to lossless compression and is consistent with the LOCO-I specification. The adaptive k mechanism partially compensates for this by selecting larger k values in high-residual regions, but the fundamental limit is imposed by the entropy of the image content.

The FPGA implementation results demonstrate that the MEDRICE core occupies a small fraction of the Zynq-7000 logic resources, leaving ample capacity for co-located application logic such as image acquisition, pre-processing, or communication stacks. The streaming pipeline architecture ensures that resource utilisation is independent of image size, providing a fixed hardware footprint regardless of the input resolution.

4.8  Design Trade-Off Analysis

The primary design trade-off in the MEDRICE architecture is between the Rice parameter adaptation speed and compression performance. A faster k adaptation (more aggressive tracking) reduces the average BPP on spatially varying images but increases the complexity of the k-update logic and may cause oscillation on noisy images. The current implementation uses the LOCO-I standard adaptation rate, which provides a balanced trade-off between adaptation speed and stability.

A secondary trade-off exists between BRAM line buffer width and maximum supported image width. The current implementation supports up to 4096 pixels per line, consuming a fixed BRAM allocation. Reducing this parameter decreases BRAM usage at the cost of a lower maximum supported resolution.

4.9  Limitations

The current implementation supports 8-bit greyscale input images only. Extension to colour (RGB) images would require either pre-processing the colour channels independently or modifying the MED predictor to exploit inter-channel correlation. Additionally, the current design does not implement the near-lossless mode of JPEG-LS, which allows a controlled maximum reconstruction error and can achieve significantly lower BPP values at the cost of not being strictly lossless.

CHAPTER 5

CONCLUSION & FUTURE WORK

5.1  Conclusion

This work presents the design and hardware implementation of the MEDRICE lossless image compression algorithm on the Xilinx Zynq-7000 SoC using the PYNQ-Z2 development platform. The proposed architecture employs the Median Edge Detector (MED) for spatial prediction and an adaptive Golomb-Rice entropy coder for efficient binary encoding of prediction residuals. A fully streaming RTL pipeline is realised in Verilog HDL, integrating a BRAM-backed line buffer, a bit-packer with flush support, and AXI4-Lite and AXI4-Stream interfaces for PS–PL communication.

The design is packaged as a reusable Vivado IP core and validated on hardware via the PYNQ framework. Compression results on a 12-image greyscale benchmark demonstrate compression ratios from 1.14 to 3.99 (average BPP = 4.74), consistent with the LOCO-I standard. Processing time scales linearly with image size, confirming the O(N) streaming architecture. FPGA resource utilisation is low, establishing the viability of the design for deployment on resource-constrained embedded platforms.

In summary, the proposed MEDRICE hardware IP core provides a hardware-efficient, low-latency, and standards-consistent lossless image compression solution suitable for integration into real-time embedded imaging pipelines on Zynq-class SoC devices.

5.2  Future Work

Several extensions to the current work are envisioned to enhance its capability and applicability:

Colour Image Support: Extend the MED predictor and Rice encoder to process three-channel RGB images, either independently per channel or with inter-channel decorrelation to exploit chromatic redundancy.

Near-Lossless Mode: Implement the JPEG-LS near-lossless mode, which bounds the maximum reconstruction error to a user-specified value ε, enabling significantly lower BPP values for applications that can tolerate minimal distortion.

Higher Throughput Architecture: Investigate multi-pixel-per-clock architectures to increase throughput beyond 1 pixel/clock, enabling compression of high-resolution video streams in real time.

On-Chip Integration with Image Sensor Interface: Integrate the MEDRICE core directly with a MIPI CSI-2 or parallel CMOS sensor interface IP core to form a complete on-chip image acquisition and compression pipeline.

Hardware Decompression Core: Design and implement a corresponding MEDRICE decompressor in RTL to enable fully on-chip lossless compression–decompression for symmetric communication applications.

Adaptive Context Modelling: Investigate the addition of a context modelling stage based on the LOCO-I context regularisation scheme to further reduce the average BPP on challenging high-entropy images.

CHAPTER 6

REFERENCES

[1]  M. J. Weinberger, G. Seroussi, and G. Sapiro, "The LOCO-I lossless image compression algorithm: Principles and standardization into JPEG-LS," IEEE Transactions on Image Processing, vol. 9, no. 8, pp. 1309–1324, Aug. 2000.

[2]  M. J. Weinberger, G. Seroussi, and G. Sapiro, "LOCO-I: A low complexity, context-based, lossless image compression algorithm," in Proc. IEEE Data Compression Conference (DCC), Snowbird, UT, USA, 1996, pp. 140–149.

[3]  ISO/IEC 14495-1:1999, "Information technology – Lossless and near-lossless compression of continuous-tone still images: Baseline," International Organization for Standardization, Geneva, Switzerland, 1999.

[4]  R. F. Rice, "Some practical universal noiseless coding techniques," Jet Propulsion Laboratory, Pasadena, CA, Tech. Rep. JPL-79-22, 1979.

[5]  S. W. Golomb, "Run-length encodings," IEEE Transactions on Information Theory, vol. 12, no. 3, pp. 399–401, Jul. 1966.

[6]  M. Abramowitz and C. Morimoto, "An ASIC implementation of the JPEG-LS lossless image compression standard," in Proc. IEEE International Symposium on Circuits and Systems (ISCAS), 2001.

[7]  X. Wu and N. Memon, "Context-based, adaptive, lossless image coding," IEEE Transactions on Communications, vol. 45, no. 4, pp. 437–444, Apr. 1997.

[8]  A. Kumar and R. Sharma, "FPGA implementation of LOCO-I lossless image compression on Xilinx Zynq-7000," in Proc. International Conference on Signal Processing and Communication (ICSC), 2019.

[9]  Xilinx Inc., "Zynq-7000 SoC Technical Reference Manual," UG585, v1.12.2, 2018. [Online]. Available: https://docs.amd.com/r/en-US/ug585-zynq-7000-SoC-TRM

[10] Xilinx Inc., "AXI Reference Guide," UG761, v14.3, 2012.

[11] Digilent Inc., "PYNQ-Z2 Reference Manual," 2019. [Online]. Available: https://digilent.com/reference/programmable-logic/pynq-z2/reference-manual

[12] A. Gersho and R. M. Gray, Vector Quantization and Signal Compression. Kluwer Academic Publishers, Boston, MA, 1992.

[13] PYNQ Project, "PYNQ: Python Productivity for Zynq," 2023. [Online]. Available: http://www.pynq.io

[14] USC-SIPI Image Database. [Online]. Available: https://sipi.usc.edu/database/